FBX - Device API (version 1.0)

Basic Auth

Username : lemco

Password : lemco!123

- Read status

http://192.168.1.210/device_api/status.php

Json reply one object (not array):

{

"system_date": "2023-07-11, 12:45",

"system_uptime": 33706601,

"application": "Running",

"cpu_temp": "33",

"cpu_load": "12%",

"memory_load": "20%",

"disk_usage": "4%",

}

Note: System uptime in seconds

application -> [“Unknown status”, “Not Running”, “Running”]

- Read Lan settings

http://192.168.1.210/device_api/lan.php

Json reply one object (not array):

{

"enable_dhcp": "0",

"ip_address": "192.168.1.105",

"network_mask": "255.255.255.0",

"gateway": "192.168.1.1",

"dns1": "8.8.8.8",

"dns2": "8.8.4.4",

"mac_address": "fa:f5:e6:12:34:56",

"web_port": "80"

}

- Read WiFi settings

http://192.168.1.210/device_api/wifi.php

Json reply one object (not array):

{

"module_present": "0",

"wifi_enable": "0",

"ssid": "SSID",

"password": "pass12345",

"mode_ap": "0",

"enable_dhcp": "0",

"ip_address": "192.168.1.105",

"network_mask": "255.255.255.0",

"gateway": "192.168.1.1",

"dns1": "8.8.8.8",

"dns2": "8.8.4.4",

"range_from": "192.168.10.2",

"range_to": "192.168.10.100",

"frequency": "2442",

"mac_address": "fa:f5:e6:12:34:56"

}

- Read info

http://192.168.1.210/device_api/info.php

Json reply one object (not array):

{

"product": "FBX-3566",

"serial_number": "1527507739",

"ethernet_mac_address": "fa:f5:e6:12:34:56",

"wifi_mac_address": "54:10:ec:6d:86:a3",

"software_version": "1.31",

}

- Execute commands

- Reset device

http://192.168.1.210/device_api/execute.php?command=device_restart

- Update LAN

http://192.168.1.210/device_api/execute.php?command=update_lan&dhcp=0&address=192.168.1.195&mask=255.255.255.0&gateway=192.168.1.1&dns1=8.8.8.8&dns2=8.8.4.4&web_port=80

or dhcp=1 (all other variables must be included, even if dchp=1)

- Update WIFI

http://192.168.1.210/device_api/execute.php?command=update_lan&dhcp=0&address=192.168.1.195&mask=255.255.255.0&gateway=192.168.1.1&dns1=8.8.8.8&dns2=8.8.4.4&web_port=80

- Play URI

http://192.168.1.210/device_api/execute.php?command=play&uri=(channel_link)

Send command to player to play specific URI

(UDP, RTP, HLS, MPEG-DASH, RTSP, RTMP, SRT, HTTP)

examples:

uri=udp://230.0.0.1:1234

uri=rtp://230.0.0.1:1234

uri=http://path-to-server/video.m3u8

uri=http://path-to-server/video.mpd

uri=rtsp://server:port/video/name

uri=rtmp://server:port/video/name

uri=srt://server:port

uri=http://path-to-file.mp4

Json reply (for all execute commands):

{

"command_valid": "1",

"return": "(message from executed command)"

}

If command_valid is “0” command has not been parsed.